DC MOTOR MODELING CODE

clc;

clear;

close all;

% DC motor parameters

R = 1;        % Armature resistance (ohm)

L = 0.5;      % Armature inductance (H)

J = 0.01;     % Rotor inertia (kg.m^2)

B = 0.1;      % Viscous friction (N.m.s)

Kt = 0.01;    % Torque constant (N.m/A)

Kb = 0.01;    % Back EMF constant (V/rad/s)

% Transfer function of DC motor (Speed / Voltage)

numerator = Kt;

denominator = [L*J  (L*B + R*J)  (R*B + Kt*Kb)];

motor_tf = tf(numerator, denominator)

% Step response of motor speed

figure;

step(motor_tf);

grid on;

title('DC Motor Speed Response');

xlabel('Time (seconds)');

ylabel('Speed (rad/sec)');

% Speed response for 5V input

figure;

step(5*motor_tf);

grid on;

title('DC Motor Speed Response for 5V Input');

% Time domain characteristics

info = stepinfo(motor_tf)

steady_state_speed = dcgain(motor_tf)

% State-space representation

A = [-R/L   -Kb/L;

Kt/J  -B/J];

B = [1/L;

0];

C = [0 1];

D = 0;

motor_ss = ss(A, B, C, D);

figure;

step(motor_ss);

grid on;

title('DC Motor State-Space Response');

% Time vector

t = linspace(0, 5, 200);

% Different input voltages

V = [1 3 5 7 10];

% Preallocate speed matrix

speed = zeros(length(V), length(t));

% Generate responses

for i = 1:length(V)

speed(i, :) = V(i) * step(motor_tf, t);

end

% Create surface plot

[T, Vgrid] = meshgrid(t, V);

figure;

surf(T, Vgrid, speed);

shading interp

xlabel('Time (seconds)');

ylabel('Input Voltage (V)');

zlabel('Motor Speed (rad/s)');

title('DC Motor Speed Response vs Time and Voltage');

OUTPUTS

motor_tf =

0.01

---------------------------

0.005 s^2 + 0.06 s + 0.1001

Continuous-time transfer function.

Model Properties

info = struct with fields:

RiseTime: 1.1351

TransientTime: 2.0652

SettlingTime: 2.0652

SettlingMin: 0.0899

SettlingMax: 0.0998

Overshoot: 0

Undershoot: 0

Peak: 0.0998

PeakTime: 3.6758